UNDERWAY pCO2 DATA FILE FORMAT

(Focuses on results from underway measurement systems.)

## I. REQUIRED ELEMENTS

## Measured atmospheric information

- Date / Time of Measurement (UTC)

- Position of measurement

  - Latitude in decimal degrees (North is positive, South is negative)

  - Longitude in decimal degrees (East is positive, West is negative)

- Mole fraction of CO2 in ambient atmosphere (µmol/mol in dry air)

- Atmospheric pressure at sea-surface pressure (hPa)

## Measured seawater information

- Date / Time of Measurement (UTC)

- Julian day

- Position of measurement

  - Latitude in decimal degrees (North is positive, South is negative)

  - Longitude in decimal degrees (East is positive, West is negative)

- Mole fraction of CO2 in air from equilibrator (µmol/mol)

- Mole fraction of H2O in air from equilibrator (µmol/mol)

- Pressure of equilibration = the pressure in the equilibration vessel (hPa)

- Temperature of equilibration = the temperature of the seawater in equilibrator at the time of equilibration (°C)

- Sea surface temperature (in situ) (°C)

- Sea surface salinity (in situ)

## II. DERIVED PARAMETERS

## Atmosphere

- x(CO2) value for the ambient atmosphere (µmol/mol in dry air); interpolated to match the date/time/position of the seawater information in this section

## Seawater

- f(CO2) for air in equilibrium with seawater at sea surface temperature (µatm). Note: the air will be at 100% humidity

- p(CO2) for air in equilibrium with seawater at sea surface temperature (µatm). Note: the air will be at 100% humidity

- x(CO2) for air in equilibrium with the seawater at sea surface temperature and 1013.25 hPa applied pressure (expressed as µmol/mol in dry air)

## III. OPTIONAL ANCILLARY INFORMATION

- Quality flags

- Detailed Ships Heading Information

- Other measurements (chemical, physical, meteorological)